# ANNEX II + III  :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of Emergency Medical Vehicles and Equipment for Emergency Medical Services (EMS) - Location - Serbia			p 1 /…

Publication reference : EuropeAid/126896/C/SUP/YU

Column 1-2 should be completed by the Contracting Authority

Column 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on his proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

LOT 1 – EMERGENCY MEDICAL VEHICLES

GENERAL REQUIREMENTS FOR THE SUPPLY

The following requirements are to be considered an integral part of the technical specifications:

- All vehicles supplied must be compliant with any required technical standard for road going passenger vehicles which are valid at the time of delivery within the Republic of Serbia. All the delivered vehicles must be left hand drive.

- Period of Execution is 120 days from contract signature for delivery to site and completion of all verification operations required for the issuance of provisional acceptance. The Contractor may arrange to make a series of Partial Deliveries (against which partial provisional acceptances certificates may be issued) subject to all items being delivered within 120 calendar days of the signature of the contract.

- Representatives of the Contracting Authority and the Beneficiary, together with the contractor’s representative, shall carry out the controls, testing and initial approval for provisional and final acceptance as provided for in the Annex I General Conditions.  The provisional and final acceptance shall take place at the final destinations indicated in the distribution list below.

- The Contractor must provide a warranty for one year (unlimited by distance) in full compliance with the provisions of Article 32 of the General Conditions and Special Conditions of the Contract. In addition. the Ministry of Health of the Republic of Serbia (the beneficiary) must be provided with the manufacturer’s standard commercial warranty.

- After-Sales Service (Please refer to Article 33 of the General and Special Conditions of the Contract): The Contractor is required to ensure that a maintenance, service and repair centre, which is authorised by the Manufacturer, and which is able to rapidly supply genuine spare parts and consumables recommended by the Manufacturer is available within the Republic of Serbia throughout the one year warranty period or the period of the manufacturer’s commercial warranty (whichever is longer). In fulfilment of this requirement with their offer Tenderers must submit a statement signed by a representative of the manufacturer:

  - providing the name and address of its authorised maintenance, service and repair centre within the Republic of Serbia

  - confirming that this authorised maintenance, service and repair centre is able to rapidly supply genuine spare parts and the consumables recommended by the Manufacturer; and

  - undertaking to ensure that the presence of a maintenance, service and repair centre authorised by it will be maintained on the territory of the Republic of Serbia throughout the one year warranty period of the period of the manufacturer’s commercial warranty (whichever is longer).

- For each delivered vehicle, according to the technical specifications, the Tenderer shall provide an instruction manual for use and maintenance of the vehicle in the Serbian language.

Delivery Address:

Ministry of Health

Nemanjina Str.  22-24
11000 Belgrade,

Serbia

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | AMBULANCES WITH SPECIAL EQUIPMENT TYPE B |  |  |  |
|  | Van type |  |  |  |
|  | Compliant with the European Standard EN 1789:2007 TYPE B |  |  |  |
|  | Fuel: Diesel |  |  |  |
|  | Front-wheel drive |  |  |  |
|  | Engine displacement: at least 2100 cc |  |  |  |
|  | Engine power: at least 74 kW |  |  |  |
|  | Engine generation – EURO 4 |  |  |  |
|  | Number of doors: 4 Doors of the patient’s compartment: There shall be a minimum of two openings – one at the rear (2 doors or tailgate) and one at the right side (sliding door with window) of the patient’s compartment. |  |  |  |
|  | Number of seats: 1 + 4 (driver, co-driver, + 3 in the patient’s compartment) |  |  |  |
|  | Wheelbase: at least 2900 mm |  |  |  |
|  | Total length of the vehicle: at least 4700 mm |  |  |  |
|  | Height of the vehicle: maximum 3000 mm , external warning light system included |  |  |  |
|  | Airbags (driver and co-driver) |  |  |  |
|  | Air conditioning and heating systems as per EN 1789: 2007 |  |  |  |
|  | ABS |  |  |  |
|  | Servo steering system |  |  |  |
|  | Central locking of all doors |  |  |  |
|  | Radio-CD |  |  |  |
|  | Alarm system |  |  |  |
|  | GPS (global positioning system with cartography of Serbia, and with last version available from the supplier of the GPS installed) |  |  |  |
|  | Electrical installation: recessed externally mounted power connector to enable external power to be provided for operations such as the following: -charging battery(ies); -operating medical devices, when installed; -operating a patient compartment heater, when installed; -operating an engine pre-heater, when installed. |  |  |  |
|  | The patient’s compartment shall be fitted with one 220 Volt socket and four x 12 Volt sockets |  |  |  |
|  | Bulkhead (partition wall): a bulkhead with a door shall separate the driver’s compartment from the patient’s compartment. It shall not be possible to drive the vehicle with the door in the open position. This door shall be secured against opening if the road ambulance is in motion. One or two windows with a minimum separation of 100 mm shall be provided in the bulkhead made of material complying with the requirements of Directive 92/22/EEC modified. The windows shall allow direct visual contact with the driver. The opening area of the window shall have a maximum area of 0,12 m². It shall be secured against self-opening and shall have an adjustable blind or other means of preventing the driver being disturbed by the light of the patient’s compartment. |  |  |  |
|  | Windows in the patient’s compartment: In the patient’s compartment, there shall be a minimum of two external windows. There shall be one on one side and the rear. The windows shall be screened to ensure patient’s privacy when required. Windows shall be made of material complying with the requirements of Directive 92/22/EEC modified. |  |  |  |
|  | The patient’s compartment shall be designed and constructed to accommodate the medical devices in accordance with the vehicle type. |  |  |  |
|  | Locked cupboards and drawers to accommodate the medical devices |  |  |  |
|  | Dimensions of the patient’s compartment: Width from the left to the right side of the ambulance, excluding roof curve, should be at least 1400 mm Length from the back side to the partition should be at least 3000 mm measured at the level of the stretcher Height from the floor to the ceiling at least 1600 mm Height from the floor to the upper stretcher surface at least 500 mm All bulges in the patient’s area should be planned and constructed to keep these minimum dimensions |  |  |  |
|  | The vehicle shall be fitted with two visual and one audible warning systems to assist emergency passage. The visual warning systems will be located on the roof. They will be integrated in the roof (in order to reduce the height of the vehicle). |  |  |  |
|  | Rotating lights are group of lights (minimum 4 peaces) placed under one cover. Light is covering all ambulance width |  |  |  |
|  | Rotating lights: front and rear lights groups |  |  |  |
|  | Blue lamps on fender, minimum 2 (alternating) |  |  |  |
|  | Electrical siren: with minimum 2 melodies |  |  |  |
|  | Ventilation system: There shall be a ventilation system which shall provide a minimum of 20 air changes per hour when the vehicle is stationary. |  |  |  |
|  | Heating-cooling system: as per EN 1789: 2007 |  |  |  |
|  | Illumination in the patient’s compartment: as per EN 1789: 2007 |  |  |  |
|  | Holding system for infusion : as per EN 1789: 2007 |  |  |  |
|  | Mounting systems: as per EN 1789: 2007 |  |  |  |
|  | Fog lamps: at least 2 pieces + 1 rear fog lamp |  |  |  |
|  | Ambulance paint and signs: the colour must be white. |  |  |  |
|  | International sign for emergency medicine must be above the windshield, on both sides of ambulance and on rear door windows – reflective colors |  |  |  |
|  | Minimum width 180mm red strip should be all around the ambulance and edge of the roof - reflective colors |  |  |  |
|  | Inscription on front side of ambulance of minimal letter size 150mm, easy readable through rear-view mirror and made of reflective foil |  |  |  |
|  | - SET FOR ENDOTRACHEAL INTUBATION - Tracheal tubes in five sizes - Laryngoscope with minimum 3 sizes of blades (from 2-4), constructed from stainless steel with standard light of minimum 2.5 V and battery supply - Laryngeal masks minimum 5 sizes - Goode Ll airways in minimum 5 sizes - Magill forceps in two sizes - Suction catheters in minimum 3 sizes and insulation foil - Bag for carrying |  |  |  |
|  | - REANIMATION SELF-EXPANDING BALOON - Opening size 15/22 mm - Masks with transparent part in minimum 3 sizes with belts for fixing the masks - Mask of semi-opened type, minimum 3 sizes and patient’s hose - Patient’s valve (autoclavable), with minimum 35 cm H2О pressure relief |  |  |  |
|  | - MOBILE OXYGEN SET - Oxygen bottle of minimum 3 l capacity with reduction valve, manometer and flow-meter, without humidifier, 0 to 15 l/min - Bag for carrying - Maintenance: high disinfection level |  |  |  |
|  | - FIXED OXYGEN SET - Oxygen bottle of minimum 10 liters, 2 pieces - Pressure reducer unit 150/5 bar, with manometer - Oxygen manifold for application unit, minimum two application units (oxygen plugs) - Flow-meter of 0-15 l/min, with humidifier, mask and oxygen hose |  |  |  |
|  | - PORTABLE АSPIRATOR AND АSPIRATION CATHETER - Supply: plug in electricity (220/230 V), and battery with installed filling element (capacity of the battery must be minimum 45 min. of work at maximum pressure), 12/24 VDC - Handle for carrying and round protection of the canister - Weight: maximum 6 kg - Maximum suction: minimum 500+ mmHg - Suction power: minimum 15 l/min - Safety: protection from overflowing, antibacterial filter additions - Transparent canister for multiple use, min. capacity 800 ml - Aspiration catheter with inbuilt holder - Analog-digital indicator of the vacuum level in mmHg (0 – 500+ mmHg) |  |  |  |
|  | - DEFIBRILATOR WITH MONITOR - External bi-phase defibrilator with pedals that can be used for selecting energy and discharge - Manual, semi-automated, sinchronous cardioversion - ECG monitoring with minimum 12 conduits - Integrated thermal printer - Battery recharger 220VAC, 12VDC/24VDC - Two batteries - Capacity of the battery must be 100 discharges with peak energy in continuity or 2 hours of continuous work - Minimum 16MB memory card - ECG display with minimum 3 curves, high resolution, adjustable: contrast, ECG amplitude and alarms, sound volume - Energy level of defibrillator from 100 to 360Ј - Loading speed to the maximum energy: 10 seconds - Maximum weight 8.5 kg - Working set minimally equipped with: 2 batteries, charger, adapters, patient’s cable, 2 rolls of thermal paper with manual in Serbian, and the bag for carrying - Working temperature: from -15°C to 40°C - Air humidity for work: up to 95% relative air humidity - Shockproof on sprinkling with water according to the European standards (moisture resistant) according to the standard, minimum IPX 2 - Pulse oxymetry with sensor, trans-cutaneous and pace-maker |  |  |  |
|  | - VACUUM SPLINTS - Vacuum splints for upper and lower limbs - Bands for fixing minimum in three sizes - Vacuum pump - Reparation set - Bag for carrying |  |  |  |
|  | - SET OF COLLARS FOR IMMOBILIZATION OF THE NECK PART OF SPINE - In six sizes - Quick and simple application - Bag for carrying - Collars that can be used during magnet resonance, RTG scanning and CT - Collars must not have latex in their composition |  |  |  |
|  | - WALL MOUNTED BLOOD PRESSURE MESURMENT DEVICE - Range 0 – 300 mmHg |  |  |  |
|  | - DEVICE FOR MEASURING BLOOD SUGAR - Test principle is determination of glucose in blood from the sample of capillary blood by reflex photometry - Measuring scope 0.6 – 33.3 mmol/l - Measuring time maximum 8 seconds - Maximum weight 100 gr - LCD monitor - Automatic switching off 1-2 min. after the last pressing of any key - Supply from the battery - Battery duration around 1.000 tests or one year - Operating temperature and air humidity according to the European standards |  |  |  |
|  | - OPENABLE STRETCHERS WITH WHEELS - The stretcher consists of detachable light Al carrier, that shall be affixed to the carriage with foldable legs - The stretcher should have 4 telescopic handles for carrying - The stretcher is on wheels - Maximum (total) weight 35 kg - Minimum length 1900 mm - Minimum width 480 mm - Minimum height with extended wheels, minimum 800 mm - Using rail system - Adjustment of the back from 0 – 900 - Constructed from anodized aluminum - Minimum carrying 150 kg - Mattress covered with material easy to wash and disinfect - Handles for mattress - Pateient fixation belts |  |  |  |
|  | - CARDIOLOGY CHAIR - Dimensions: - Height with backrest minimum 900 mm - Minimum width 500 mm - Seat depth minimum 450 mm - Height of the back minimum 450 mm - Length of the arm-rests on the chair backrest minimum 250mm - Height of the arm-rests on the chair backrest from the base minimum 750mm - Length of lower arm-rests (maximally extended) minimum 400mm - Maximum weight 10 kg - Minimum carrying 150 kg - Constructed from anodized aluminum - Two (2) wheels - Belts for fixing the patient to the chair - Cover material must be easily washed and disinfected - Cardiology chair should have 4 foldable-telescopic handles for carrying |  |  |  |
|  | - PORTABLE 3 CHANEL ECG DEVICE - With electricity plug in suplly, and battery supply integrated into device - Amlitude selection: 5-10-20 mm/mV - Printing speed 5-25-50 mm/s - Integrated three channel thermal printer (minimum width 90mm) - Posibility of last ECG printing - Operation mode: automatic (simultaneous/sequential) and manuel - QRS beeper - Capacity of the battery 2 hours of continuous work or 50 ECG’s - Maximum weight of the device with battery 3kg - Protection from defibrillation - Protective suitcase |  |  |  |
|  | - STRETCHER FOR MULTIPLE TRAUMA (SPOON - STRETCHER) - Maximum weight 10 kg - Minimum carrying 150 kg - Contructed from anodized aluminum - Lenght minimum 1600 mm and maximum 2050 - Minimum width 425 mm - Belts for fixing the patient, min. 3 pieces and head fixing device - The stretcher has been properly placed and affixed depending on the patient area |  |  |  |
|  | - ADDITIONAL OPENABLE STRETCHER - Dimensions: length – minimum 2000 mm, length of folded stretchers minimum – 1000 mm, width – minimum 480 mm, minimum height– 130 mm, height of folded stretchers minimum 160 mm - Minimum carrying 150 kg - Constructed from anodized aluminum - Maximum stretcher weight 6 kg |  |  |  |
|  | - EMERGENCY DELIVERY KIT |  |  |  |
|  | - COMMUNICATIONS - Car radio station (fixed with minimum 10 channels) |  |  |  |
|  | Certificate of compliance with EN 1789:2007 |  |  |  |
|  | TRAINING: training on site for this item: 8 days - including fees, transport and accommodation cost of the personnel of the supplier. The Tenderer shall provide training for four people in each location in the use and application of the equipment and car maintenance. The Tenderer shall provide a detailed training plan. The training shall be given in Serbian language. Course material and reference books shall be provided in English and Serbian. Location of the trainings: 2 days in Belgrade + 2 days in Nis, 2 days in Novi Sad, 2 days in Kragujevac: total 8 days. |  |  |  |
|  | QUANTITY: 95 |  |  |  |

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 2 | AMBULANCES WITH SPECIAL EQUIPMENT TYPE C |  |  |  |
|  | Van type |  |  |  |
|  | Compliant with the European Standard EN 1789:2007 TYPE C |  |  |  |
|  | Fuel: Diesel |  |  |  |
|  | Front-wheel drive |  |  |  |
|  | Engine displacement: at least 2100 cc |  |  |  |
|  | Engine power: at least 74 kW |  |  |  |
|  | Engine generation – EURO 4 |  |  |  |
|  | Number of doors: 4 Doors of the patient’s compartment: There shall be a minimum of two openings – one at the rear (2 doors or tailgate) and one at the right side (sliding door with window) of the patient’s compartment. |  |  |  |
|  | Number of seats: 1 + 4 (driver, co-driver, + 3 in the patient’s compartment) |  |  |  |
|  | Wheelbase: at least 2900 mm |  |  |  |
|  | Total length of the vehicle: at least 4700 mm |  |  |  |
|  | Height of the vehicle: maximum 3000 mm , external warning light system included |  |  |  |
|  | Airbags (driver and co-driver) |  |  |  |
|  | Air conditioning and heating systems as per EN 1789: 2007 |  |  |  |
|  | ABS |  |  |  |
|  | Servo steering system |  |  |  |
|  | Central locking of all doors |  |  |  |
|  | Radio-CD |  |  |  |
|  | Alarm system |  |  |  |
|  | GPS (global positioning system with cartography of Serbia, and with last version available from the supplier of the GPS installed) |  |  |  |
|  | Electrical installation: recessed externally mounted power connector to enable external power to be provided for operations such as the following: -charging battery(ies); -operating medical devices, when installed; -operating a patient compartment heater, when installed; -operating an engine pre-heater, when installed. |  |  |  |
|  | The patient’s compartment shall be fitted with one 220 Volt socket and four x 12 Volt sockets |  |  |  |
|  | Bulkhead (partition wall): a bulkhead with a door shall separate the driver’s compartment from the patient’s compartment. It shall not be possible to drive the vehicle with the door in the open position. This door shall be secured against opening if the road ambulance is in motion. One or two windows with a minimum separation of 100 mm shall be provided in the bulkhead made of material complying with the requirements of Directive 92/22/EEC modified. The windows shall allow direct visual contact with the driver. The opening area of the window shall have a maximum area of 0,12 m². It shall be secured against self-opening and shall have an adjustable blind or other means of preventing the driver being disturbed by the light of the patient’s compartment. |  |  |  |
|  | Windows in the patient’s compartment: In the patient’s compartment, there shall be a minimum of two external windows. There shall be one on one side and the rear. The windows shall be screened to ensure patient’s privacy when required. Windows shall be made of material complying with the requirements of Directive 92/22/EEC modified. |  |  |  |
|  | The patient’s compartment shall be designed and constructed to accommodate the medical devices in accordance with the vehicle type. |  |  |  |
|  | Locked cupboards and drawers to accommodate the medical devices |  |  |  |
|  | Dimensions of the patient’s compartment: Width from the left to the right side of the ambulance, excluding roof curve, should be at least 1400 mm Length from the back side to the partition should be at least 3000 mm measured at the level of the stretcher Height from the floor to the ceiling at least 1800 mm Height from the floor to the upper stretcher surface at least 500 mm All bulges in the patient’s area should be planned and constructed to keep these minimum dimensions |  |  |  |
|  | The vehicle shall be fitted with two visual and one audible warning systems to assist emergency passage. The visual warning systems will be located on the roof. They will be integrated in the roof (in order to reduce the height of the vehicle). |  |  |  |
|  | Rotating lights are group of lights (minimum 4 peaces) placed under one cover. Light is covering all ambulance width |  |  |  |
|  | Rotating lights: front and rear lights groups |  |  |  |
|  | Blue lamps on fender, minimum 2 (alternating) |  |  |  |
|  | Electrical siren: with minimum 2 melodies |  |  |  |
|  | Ventilation system: There shall be a ventilation system which shall provide a minimum of 20 air changes per hour when the vehicle is stationary. |  |  |  |
|  | Heating-cooling system: as per EN 1789: 2007 |  |  |  |
|  | Illumination in the patient’s compartment: as per EN 1789: 2007 |  |  |  |
|  | Holding system for infusion : as per EN 1789: 2007 |  |  |  |
|  | Mounting systems: as per EN 1789: 2007 |  |  |  |
|  | Fog lamps: at least 2 pieces + 1 rear fog lamp |  |  |  |
|  | Ambulance paint and signs: the colour must be white |  |  |  |
|  | International sign for emergency medicine must be above the windshield, on both sides of ambulance and on rear door windows – reflective colors |  |  |  |
|  | Minimum width 180mm red strip should be all around the ambulance and edge of the roof - reflective colors |  |  |  |
|  | Inscription on front side of ambulance of minimal letter size 150mm, easy readable through rear-view mirror and made of reflective foil |  |  |  |
|  | - SET FOR ENDOTRACHEAL INTUBATION - Tracheal tubes in five sizes - Laryngoscope with minimum 3 sizes of blades (from 2-4), constructed from stainless steel with standard light of minimum 2.5 V and battery supply - Laryngeal masks minimum 5 sizes - GoodeLl airways in minimum 5 sizes - Magill forceps in two sizes - Suction catheters in minimum 3 sizes and insulation foil - Bag for carrying |  |  |  |
|  | - REANIMATION SELF-EXPANDING BALOON - Opening size 15/22 mm - Masks with transparent part in minimum 3 sizes with belts for fixing the masks - Mask of semi-opened type, minimum 3 sizes and patient’s hose - Patient’s valve (autoclavable), with minimum 35 cm H2О pressure relief |  |  |  |
|  | - MOBILE OXYGEN SET - Oxygen bottle of minimum 3 l capacity with reduction valve, manometer and flow-meter, without humidifier, 0 to 15 l/min - Bag for carrying - Maintenance: high disinfection level |  |  |  |
|  | - FIXED OXYGEN SET - Oxygen bottle of minimum 10 liters, 2 pieces - Pressure reducer unit 150/5 bar, with manometer - Oxygen manifold for application unit, minimum two application units (oxygen plugs) - Flow-meter of 0-15 l/min, with humidifier, mask and oxygen hose |  |  |  |
|  | - PORTABLE RESPIRATOR (VENTILATOR) - Produced according EU Directive 93/42/EEC with CE sign - Temperature/humidity operation mode: -150 C till +500C, relative humidity 20 - 95% - Oxygen driven with supply pressure 2.7 till 6 bar, no electrical power needed for operation - Ventilation mode: IPPV - Possibility PEEP regulation with external valve - Parameters adjustable: frequency 5-50 per min., minute volume 3-20 lit., maximal pressure in breathing pathways 25-50 mbar - Concentration Oxygen operation modes with 60% and 100% Oxygen - Pressure display on front side of device - Alarms for Paw low (disconnect) Paw high (stenosis) Supply pressure low - Oxygen bottle of 2 lit, with valve - Connectors for central gas system - Carrying device - Total weight of device with carrying system maximum 8 kg |  |  |  |
|  | - PORTABLE АSPIRATOR AND АSPIRATION CATHETER - Supply: plug in electricity (220/230 V), and battery with installed filling element (capacity of the battery must be minimum 45 min. of work at maximum pressure), 12/24 VDC - Handle for carrying and round protection of the canister - Weight: maximum 6 kg - Maximum suction: 500+ mmHg - Suction power: 15 l/min - Safety: protection from overflowing, antibacterial filter additions - Transparent canister for multiple use, min. capacity 800 ml - Aspiration catheter with inbuilt holder - Analog-digital indicator of the vacuum level in mmHg (0 – 500+ mmHg) |  |  |  |
|  | - DEFIBRILATOR WITH MONITOR - External bi-phase defibrilator with pedals that can be used for selecting energy and discharge - Manual, semi-automated, sinchronous cardioversion - ECG monitoring with minimum 12 conduits - Integrated thermal printer - Battery recharger 220VAC, 12VDC/24VDC - Two batteries - Capacity of the battery must be 100 discharges with peak energy in continuity or 2 hours of continuous work - Minimum 16MB memory card - ECG display with minimum 3 curves, high resolution, adjustable: contrast, ECG amplitude and alarms, sound volume - Energy level of defibrillator from 100 to 360Ј - Loading speed to the maximum energy: 10 seconds - Maximum weight 8.5 kg - Working set minimally equipped with: 2 batteries, charger, adapters, patient’s cable, 2 rolls of thermal paper with manual in Serbian, and the bag for carrying - Working temperature: from -15°C to 40°C - Air humidity for work: up to 95% relative air humidity - Shockproof on sprinkling with water according to the European standards (moisture resistant) according to the standard, minimum IPX 2 - Pulse oxymetry with sensor, trans-cutaneous and pace-maker |  |  |  |
|  | - VACUUM SPLINTS - Vacuum splints for upper and lower limbs - Bands for fixing minimum in three sizes - Vacuum pump - Reparation set - Bag for carrying |  |  |  |
|  | - VACUUM MATTRES - Size minimum2000 x 800 mm - Maximum weight 6.5 kg - Reparation set - Bag for carrying and vacuum pump |  |  |  |
|  | - SET OF COLLARS FOR IMMOBILIZATION OF THE NECK PART OF SPINE - In six sizes - Quick and simple application - Bag for carrying - Collars that can be used during magnet resonance, RTG scanning and CT - Collars must not have latex in their composition |  |  |  |
|  | - WALL MOUNTED BLOOD PRESSURE MESURMENT DEVICE - Range 0 – 300 mmHg |  |  |  |
|  | - DEVICE FOR MEASURING BLOOD SUGAR - Test principle is determination of glucose in blood from the sample of capillary blood by reflex photometry - Measuring scope 0.6 – 33.3 mmol/l - Measuring time maximum 8 seconds - Maximum weight 100 gr - LCD monitor - Automatic switching off 1-2 min. after the last pressing of any key - Supply from the battery - Battery duration around 1.000 tests or one year - Operating temperature and air humidity according to the European standards |  |  |  |
|  | - OPENABLE STRETCHERS WITH WHEELS - The stretcher consists of detachable light Al carrier, that shall be affixed to the carriage with foldable legs - The stretcher should have 4 telescopic handles for carrying - The stretcher is on wheels - Maximum (total) weight 35 kg - Minimum length 1900 mm - Minimum width 480 mm - Minimum height with extended wheels, minimum 800 mm - Using rail system - Adjustment of the back from 0 – 900 - Constructed from anodized aluminum - Minimum carrying 150 kg - Mattress covered with material easy to wash and disinfect - Handles for mattress - Pateient fixation belts |  |  |  |
|  | - CARDIOLOGY CHAIR - Dimensions: - Height with backrest minimum 900 mm - Minimum width 500 mm - Seat depth minimum 450 mm - Height of the back minimum 450 mm - Length of the arm-rests on the chair backrest 250mm - Height of the arm-rests on the chair backrest from the base 750mm - Length of lower arm-rests (maximally extended) minimum 400mm - Maximum weight 10 kg - Minimum carrying 150 kg - Constructed from anodized aluminum - Two (2) wheels - Belts for fixing the patient to the chair - Cover material must be easily washed and disinfected - Cardiology chair should have 4 foldable-telescopic handles for carrying |  |  |  |
|  | - PORTABLE 3 CHANEL ECG DEVICE - With electricity plug in suplly, and battery supply integrated into device - Amlitude selection: 5-10-20 mm/mV - Printing speed 5-25-50 mm/s - Integrated three channel thermal printer (minimum width 90mm) - Posibility of last ECG printing - Operation mode: automatic (simultaneous/sequential) and manuel - QRS beeper - Capacity of the battery 2 hours of continuous work or 50 ECG’s - Maximum weight of the device with battery 3kg - Protection from defibrillation - Protective suitcase |  |  |  |
|  | - STRETCHER FOR MULTIPLE TRAUMA (SPOON - STRETCHER) - Maximum weight 10 kg - Minimum carrying 150 kg - Contructed from anodized aluminum - Lenght minimum 1600 mm and maximum 2050 - Minimum width 425 mm - Belts for fixing the patient, min. 3 pieces and head fixing device - The stretcher has been properly placed and affixed depending on the patient area |  |  |  |
|  | - ADDITIONAL OPENABLE STRETCHER - Dimensions: length – minimum 2000 mm, length of folded stretchers minimum – 1000 mm, width – minimum 480 mm, minimum height– 130 mm, height of folded stretchers minimum 160 mm - Minimum carrying 150 kg - Constructed from anodized aluminum - Maximum stretcher weight 6 kg |  |  |  |
|  | - EMERGENCY DELIVERY KIT |  |  |  |
|  | - COMMUNICATIONS - Car radio station (fixed with minimum 10 channels) |  |  |  |
|  | Certificate of compliance with EN 1789:2007 |  |  |  |
|  | TRAINING: training on site for this item: 8 days - including fees, transport and accommodation cost of the personnel of the supplier. The Tenderer shall provide training for four people in each location in the use and application of the equipment and car maintenance. The Tenderer shall provide a detailed training plan. The training shall be given in Serbian language. Course material and reference books shall be provided in English and Serbian. Location of the trainings: 2 days in Belgrade + 2 days in Nis, 2 days in Novi Sad, 2 days in Kragujevac: total 8 days. |  |  |  |
|  | QUANTITY: 120 |  |  |  |

# LOT 2: EMERGENCY MEDICAL SERVICE TRAINING EQUIPMENT

GENERAL REQUIREMENTS FOR THE SUPPLY

The following requirements are to be considered an integral part of the technical specifications:

- Period of Execution is 120 days from contract signature for delivery to site and completion of all verification operations required for the issuance of provisional acceptance. The Contractor may arrange to make a series of Partial Deliveries (against which partial provisional acceptances certificates may be issued) subject to all items being delivered within 120 calendar days of the signature of the contract.

- Representatives of the Contracting Authority and the Beneficiary, together with the contractor’s representative, shall carry out the controls, testing and initial approval for provisional and final acceptance as provided for in the Annex I General Conditions.  The provisional and final acceptance shall take place at the final destinations indicated in the distribution list below.

- The Contractor must provide a warranty for one year in full compliance with the provisions of Article 32 of the General Conditions and Special Conditions of the Contract. In addition. the Ministry of Health of the Republic of Serbia (the beneficiary) must be provided with the manufacturer’s standard commercial warranty.

- After-Sales Service (Please refer to Article 33 of the General and Special Conditions of the Contract): The Contractor is required to ensure that a maintenance, service and repair centre, which is authorised by the Manufacturer, and which is able to rapidly supply genuine spare parts and consumables recommended by the Manufacturer is available within the Republic of Serbia throughout the one year warranty period or the period of the manufacturer’s commercial warranty (whichever is longer). In fulfilment of this requirement with their offer Tenderers must submit a statement signed by a representative of the manufacturer:

  - providing the name and address of its authorised maintenance, service and repair centre within the Republic of Serbia

  - confirming that this authorised maintenance, service and repair centre is able to rapidly supply genuine spare parts and the consumables recommended by the Manufacturer; and

  - undertaking to ensure that the presence of a maintenance, service and repair centre authorised by it will be maintained on the territory of the Republic of Serbia throughout the one year warranty period of the period of the manufacturer’s commercial warranty (whichever is longer).

- For each delivered item, according to the technical specifications, the Tenderer shall provide an instruction manual for use and maintenance of the vehicle in the Serbian language.

Delivery Address:

Ministry of Health

Nemanjina Str.  22-24
11000 Belgrade,

Serbia

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | TRAINING MANIKIN-CHILD SIZE |  |  |  |
|  | Child-size training manikin.(+/- 4 to 7 years old) |  |  |  |
|  | Training for CPR (Cardiopulmonary resuscitation) and rescue |  |  |  |
|  | Manikin to learn the techniques of opening the airway |  |  |  |
|  | Tilt of head, jaw and chin |  |  |  |
|  | Simulation of chest ventilation and compression |  |  |  |
|  | Simulation of carotids pulse |  |  |  |
|  | Disposable airways |  |  |  |
|  | Feedback of CPR performance |  |  |  |
|  | Carrying case |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 2 | TRAINING MANIKIN-BABY SIZE-CPR |  |  |  |
|  | Baby-size training manikin. |  |  |  |
|  | Training for CPR (Cardiopulmonary resuscitation) and rescue |  |  |  |
|  | Manikin to learn the techniques of opening the airway |  |  |  |
|  | Tilt of head, jaw and chin |  |  |  |
|  | Simulation of chest ventilation and compression |  |  |  |
|  | Simulation of brachial pulse |  |  |  |
|  | Disposable airways |  |  |  |
|  | Feedback of CPR performance |  |  |  |
|  | Carrying case |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 3 | TRAINING MANIKIN-ADULT SIZE |  |  |  |
|  | Adult-size training manikin |  |  |  |
|  | Training for CPR (Cardiopulmonary resuscitation) and use of defibrillators |  |  |  |
|  | The manikin must be able to accept high voltage shock with a real defibrillator and correctly simulate cardiac rhythms |  |  |  |
|  | Manikin to learn the techniques of opening the airway |  |  |  |
|  | Tilt of head, jaw and chin |  |  |  |
|  | Simulation of chest ventilation and compression |  |  |  |
|  | Simulation of carotids pulse |  |  |  |
|  | Disposable airways |  |  |  |
|  | Feedback of CPR performance |  |  |  |
|  | Carrying case |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 4 | TRAINING manikin for rescue, extrication, and trauma care |  |  |  |
|  | Adult-size training manikin |  |  |  |
|  | Training for rescue, trauma care and extrication |  |  |  |
|  | Intubation head for airway management |  |  |  |
|  | Trauma intubation head |  |  |  |
|  | Facial and cranial trauma head |  |  |  |
|  | Realistic articulation for application of cervical collars, splints and traction, or application to spine-board |  |  |  |
|  | Bullet wound chest module |  |  |  |
|  | Realistic articulation |  |  |  |
|  | Carrying case |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 5 | TRAINING MANIKIN-PNEUMOTHORAX |  |  |  |
|  | Training for chest decompression |  |  |  |
|  | Torso adult-size training manikin with bilateral tension pneumo-thoracic and decompression portals |  |  |  |
|  | Carrying case adult-size training |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 6 | TRAINING MANIKIN-CRICOID STICK HEAD |  |  |  |
|  | Training for needle and surgical crico-thyrothomy |  |  |  |
|  | Head with rigid and soft trachea, simulation of lung and replaceable neck skin |  |  |  |
|  | Base support |  |  |  |
|  | Carrying case |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 7 | TRAINING MANIKIN-HEIMLICH ABDOMINAL THRUST |  |  |  |
|  | Training for Heimlich abdominal thrust |  |  |  |
|  | Torso adult-size training manikin |  |  |  |
|  | Possibility of training with simulated food boluses |  |  |  |
|  | Carrying case |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 8 | TRAINING MANIKIN-BABY SIZE-ALS |  |  |  |
|  | Baby-size training manikin for advanced life support |  |  |  |
|  | Training for resuscitation, with airway anatomy |  |  |  |
|  | Manikin to learn the techniques of airway management, CPR, ECG and vascular access |  |  |  |
|  | Training for intubation and ventilation |  |  |  |
|  | Training for bone marrow aspiration |  |  |  |
|  | Rhythm simulator |  |  |  |
|  | Carrying case |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 9 | TRAINING MANIKIN-NEWBORN |  |  |  |
|  | Newborn-size training manikin to learn the techniques of vascular access |  |  |  |
|  | Equipped with jugular, temporal, saphenous and popliteal veins |  |  |  |
|  | Training for venipuncture, insertion of a catheter, umbilical catheterization |  |  |  |
|  | Carrying case |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 10 | TRAINING MANIKIN-AIRWAY MANAGEMENT |  |  |  |
|  | Training for airway management |  |  |  |
|  | Head, or upper torso and head adult-size training manikin |  |  |  |
|  | Manikin to practice the techniques of intubation |  |  |  |
|  | For training with laryngoscopes, training of tracheal intubation, introduction of a laryngeal mask, fiberoptic intubation, insertion of tubes |  |  |  |
|  | Movement of the head, cervical spine and jaw. |  |  |  |
|  | Lifting and tilting of the head. |  |  |  |
|  | Head on a stable base. |  |  |  |
|  | Carrying case. |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 11 | TRAINING MANIKIN-ARM |  |  |  |
|  | Training for intravenous therapy |  |  |  |
|  | Rotating arm with representation of veins |  |  |  |
|  | For training with insertion of catheters and cannulas, infusion of fluids, injection of liquids, blood sampling and pulse measurement |  |  |  |
|  | Infusion stand |  |  |  |
|  | Complete with syringes and consumables for start-up |  |  |  |
|  | Arm on a stable base |  |  |  |
|  | Carrying case |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 12 | TRAINING MANIKIN-ALS SIMULATION |  |  |  |
|  | Adult-size training manikin |  |  |  |
|  | Training for advanced life support |  |  |  |
|  | Full body for training in CPR (Cardiopulmonary resuscitation), ACLS (Advanced cardiac life support), bleeding control and trauma |  |  |  |
|  | Carrying case |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 13 | CPR MASK |  |  |  |
|  | Mask with one-way valve |  |  |  |
|  | Hydrophobic filter to avoid passage of liquids |  |  |  |
|  | Oxygen inlet |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 14 | TUBES AND MASKS |  |  |  |
|  | Laryngeal tubes- quantity: 5 |  |  |  |
|  | Laryngeal mask- quantity: 10 |  |  |  |
|  | Nasopharyngeal tube- quantity: 5 |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 15 | BALLOON |  |  |  |
|  | Self-re-inflating bag with non-re-breathing valves to provide positive pressure ventilation during resuscitation with oxygen or air |  |  |  |
|  | Delivers a volume of +/- 1 litre |  |  |  |
|  | QUANTITY: 8 |  |  |  |
| 16 | IMMOBILIZATION EQUIPMENT AND MISCELLANEOUS |  |  |  |
|  | Neck collar: orthopaedic piece to support a patient's spinal cord and head (for adults)- quantity : 2 |  |  |  |
|  | Leg immobilizer: orthopaedic piece to support a patient's leg (for adults)- quantity : 4 |  |  |  |
|  | Arm immobilizer: orthopaedic piece to support a patient's leg (for adults)-quantity : 4 |  |  |  |
|  | Inflatable mattress- length +/- 2 meters- quantity : 1 |  |  |  |
|  | Syringe, glass and stainless steel, capacity +/- 5 ml- quantity : 5 |  |  |  |
|  | Syringe, glass and stainless steel, capacity +/- 50 ml-quantity : 5 |  |  |  |
|  | QUANTITY: 8 |  |  |  |
|  | TRAINING: training on site for this LOT: 8 days - including fees, transport and accommodation cost of the personnel of the supplier. The Tenderer shall provide training for four people in each location in the use and application of the equipment. The Tenderer shall provide a detailed training plan. The training shall be given in Serbian language. Course material and reference books shall be provided in English and Serbian. Location of the trainings: 2 days in Belgrade + 2 days in Nis, 2 days in Novi Sad, 2 days in Kragujevac: total 8 days. |  |  |  |