# ANNEX II + III:	TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title	Supply of Security Equipment Supporting Implementation of ECAA Agreement in Serbia

Publication reference	EuropeAid/128957/C/SUP/RS
07SER01/26/21

Columns 1–2 should be completed by the Contracting Authority

Columns 3–4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex II – the Technical Specifications

General requirements

- The specifications given in the following are requirements based on the respective item’s projected function or purpose. Characteristics and features deviating from particular technical solutions, figures or values given in the specifications (unless exceeding the requirements), due to the particular design of the actually offered product, are acceptable, as long as the functional capacity is at least equivalent to the given specification and permissible under the requirements according to EU technical regulations, or standards quoted in these Technical Specifications.

Full compatibility with other items specified, or equipment already available at the beneficiary institution when described in these Technical Specifications, is mandatory. Accuracy and Class shall always be as required or better. Adequate documentation to assess the respective capacity, quality, compatibility and inter-connectivity of the product offered must be included with the offer.

Any specification referring to or suggesting a particular product or manufacturer, in particular to types, models and brand names, are always to be understood as “or equivalent”. Where equivalency shall be subject to technical evaluation, the respective documentation of equivalency, and – if appropriate – an assessment by an independent party shall be provided with the offer.

- All equipment shall be in conformity with the applicable EU technical regulations for safety and health. Evidence for compliance has to be provided (such as CE marking, manufacturer declaration, relevant marks/signs or copies of certificates) for such equipment.

- The equipment shall be capable of withstanding power failures and equipment should not be damaged due to 'trip outs'. The normal voltage and frequency conditions available at the site are AC 220-240 V and 50 Hz.

The supplies shall include all accessories necessary for proper operation of the equipment such as, but not limited to power cables, interconnecting cables, adapters, connectors, input devices and power adapters and transport cases for mobile equipment.

The requested accessories are only those needed for the equipment to be put into operation. They shall be offered free of charge, as inseparable part of the equipment.

- External working conditions:

All equipment shall be rated for operation at an ambient temperature from 0° to +40 ºC, and a relative humidity from 5–90 %.

- Requirements for installation, testing, demonstration and training.

Installation, testing, demonstration and training shall be completed within the time frame set out in the Special Conditions of Supply Contracts included in the Tender Dossier. At the end of this period, all equipment supplied including hardware and software shall be fully operational and the staff chosen by the beneficiary will be given full training for safe and efficient use of the equipment.

Provisional acceptance by the Beneficiary will depend both on the supply of all the equipment and on the requested installation, testing, demonstration and training having been satisfactorily completed. The Supplier shall describe in his technical proposal, how, related to time, it suggest carrying out the required installation, testing, demonstration and training activities and include a bar chart (GANTT chart) showing the allocated time and the time of performance.

All work done by the Supplier and any of his agents shall comply with EC regulations and international best practice with respect to health, safety and the environment.

The Supplier shall make provision of imparting training to the Beneficiary’s staff on operation and use of equipment, its accessories and software for the intended applications, troubleshooting, repair and maintenance.

As part of installation work, the supplier shall calibrate, test and run all the equipment installed or delivered and demonstrate to the beneficiary that the equipment is fully functional.

Reagents, packages, calibration/buffer solvents/materials, etc. - required for start up and commissioning shall be provided by the supplier.

Installation and testing shall be done immediately before the training in operation so that the beneficiary’s staff can gain first-hand experience in installation and testing techniques. The training program shall be included as an integral component of the offer. The training shall take place at the Beneficiaries’ premises.

- The technical offer shall contain technical documentation of all equipment in English language, including the technical specification of the manufacture (data sheets).

Full instrument documentation, including description of scientific method, as well as Manuals for equipment operation and regular maintenance in English language shall be included with the supplies.

- Warranty period.

During the warranty period, the Supplier shall provide permanent “hot-line” (by phone, fax and email) technical support to the Beneficiary with onsite response time of 48 hours or less.

Warranty period for all supplies shall be 1 year from the date of equipment commissioning.

The commercial warranty of at least two years for all supplies will be provided, if not stated differently in the Technical Specification.

- After-sales service

The Contractor shall provide or secure the provision of a reliable after-sales service, thereafter guaranteeing maintenance, technical back up and the rapid replenishment of spare parts and consumables for the offered goods for the period of 7 (seven) years after provisional acceptance.

In the event of termination of production of the spare parts, advance notification to the Beneficiary will be sent to allow procurement of the parts required.

- Quality assurance

All manufacturers shall be ISO 9001 certified. A copy of ISO certificate shall be delivered with the offer.

- Proof of compliancy on safety and security standards and regulations that need to be provided with the tender

- For x-ray machines: EC 98/37, 89/336, 73/23, ANSI43.17or equivalent

- For walk through metal detectors declaration for the conformity for IP20 or better.

- Place of delivery

| Item No. | Place of delivery per Beneficiary |  |  |
| --- | --- | --- | --- |
|  | Belgrade Airport (qty) | Civil Aviation Directorate (qty) | Nis Airport (qty) |
| 1 |  | 1 |  |
| 2 | 1 |  |  |
| 3 |  | 1 |  |
| 4 |  |  | 1 |
| 5 | 1 |  |  |
| 6 | 1 |  | 2 |
| 7 |  | 1 |  |
| 8 |  |  | 1 |
| 9 | 1 |  |  |

- Period of delivery

| Lot 1 | 90 days |
| --- | --- |
| Lot 2 | 180 days |
| Lot 3 | 90 days |
| Lot 4 | 180 days |

Annex III – the Contractor's Technical Offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority showing the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

LOT No.1

| Item Number | Specifications Required | Specifications Offered | Notes, remarks, refs to documentation | Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | COMPUTER BASED X-RAY SCREENER TRAINING SOFTWARE |  |  |  |
| 1.1. | Principle of operation Software aimed for initial and ongoing training and testing of aviation security screeners. Computer Based Training (CBT) – a specific training and e-learning program developed for training operators / screeners of hold baggage, hand luggage and cargo- operating screening machines. |  |  |  |
| 1.2. | Technical specifications |  |  |  |
| 1.2.1 | The provided system for training shall use real x-ray images with real threat images to ensure realistic image and right scaling of items, all bags should be 100% realistic if an expert would view the images. |  |  |  |
| 1.2.2 | For security reasons the operating system for training shall be isolated from internet. Training network shall be based on LAN. |  |  |  |
| 1.2.3 | The provided system for training shall support at least two major x-ray machine providers commercially available. It shall support equipment currently used in the Serbian Airports. |  |  |  |
| 1.2.4 | For every single user (trainee and instructor) personalized password shall be assigned to ensure easy access for users and to allow instructor to check trainee's performance and activity. |  |  |  |
| 1.2.5 | The provided system for training shall have a bookmark facility for ongoing training. |  |  |  |
| 1.2.6 | The provided system for training shall have the features of archiving results and be able to automatically keep records for each screener, with result reporting ability. |  |  |  |
| 1.2.7 | Software function in the provided system for training shall limit trainee’s access to performance results and give the full rights only to authorized instructors. |  |  |  |
| 1.2.8 | The provided system for training shall be able to use x-ray images (raw data) captured from x-ray equipment used in Serbian Airports. |  |  |  |
| 1.2.9 | Instructor shall be able to change any of the x-ray images, delete images, add images, and change learning instructions and threat items. |  |  |  |
| 1.2.10 | The selected manufacturer shall be able to adjust decision time (time out feature) according to customer’s requirements on the time of installation. |  |  |  |
| 1.2.11 | All major functions available on x-ray machines keyboard shall be able to be used by trainee on training session and all images should appear 100% as on real x-ray machine. Following functions are mandatory: - Standard view / NORM, - Black & White, - Organic stripping / Inorganic material, - Sensitivity / Cristal clear, - HIGH, - Negative / Invert, - Organic Only+ / Organic material. |  |  |  |
| 1.2.11.1 | Generic keyboards and generic image manipulation functions are not allowed, all keyboards provided shall support real x-ray machines Smiths Heimann and Rapiscan x-ray machines. |  |  |  |
| 1.2.12 | Additional training modules shall be able to be attached separately to the system provided for training to allow further system expansion. (For example, x-ray awareness, weapon training, dangerous objects training, aviation security training, explosives training.) |  |  |  |
| 1.2.13 | Image library shall have at least 700 real x-ray images for each x-ray machine supported (full learning objects including real images of the x-ray image). These images shall include threat items of common and rare kind (e.g., components of IED’s, weapons, knives, stun guns, DGR, batons etc.) |  |  |  |
| 1.2.14 | The system provided for training shall support at least three ways of training and testing. (e.g. item pinpointing, threat recognition, material recognition, final testing.) Instructor shall be able to switch one training mode to other. |  |  |  |
| 1.2.15 | The system provided for training shall enable the trainee to check the content of current x-ray image displayed (comparing x-ray image to real image). |  |  |  |
| 1.2.16 | The system provided for training shall allow the training of groups of trainees and to allocate different lessons to different groups (e.g. new comers, refresher course, cargo, hand luggage, hold baggage.) |  |  |  |
| 1.2.17 | The software constituting the system provided for training shall be the latest licensed version on the market from software producer at the time of commissioning. |  |  |  |
| 1.2.18 | The operating system provided for training shall be able to run on PC´s that are at sale as “Commercial off-the-Shelf” (COTS) products” - Operating with (Microsoft based) Windows XP or Vista. |  |  |  |
| 1.2.19 | A list of hardware and software requirements shall be provided with the offer. |  |  |  |
| 1.2.21 | Remote access to the system shall be possible for maintenance and administration purposes via secured and fixed IP-addresses. Internet access shall be limited only to maintenance and administrative purposes. |  |  |  |
| 1.3 | Capacity |  |  |  |
| 1.3.1 | System shall allow five trainees to use the system simultaneously for training. |  |  |  |
| 1.3.2 | System shall have, in addition to five trainees stations also an additional server and administration station that can be used without disturbing trainee units. | . |  |  |

LOT No. 2

| Item Number | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 2 | X-RAY SCREENING SYSTEM FOR HAND LUGGAGE WITH TIP |  |  |  |
| 2.1. | Equipment description Detection systems that use x-ray generator to interrogate the material in question and detect the response of all present materials. |  |  |  |
| 2.2. | Technical specifications |  |  |  |
| 2.2.1. | Equipment shall comply with all requirements laid down by ECAC (DOC30 including annexes) and European Union (EC781/2005) for x-ray machines used to screen passengers hand luggage and personal belongings. |  |  |  |
| 2.2.2. | In addition to ECAC and EU requirements mentioned at least following technical requirements shall be met: |  |  |  |
| 2.2.2.1 | Tunnel size shall have the dimensions described below; Minimum 600 mm x 400 mm. Maximum 640 mm x 455 mm. |  |  |  |
| 2.2.1.2 | TIP system shall be installed (with optional capability to provide individual access code for each user or to use single code for all operators according to the need of airport authority). |  |  |  |
| 2.2.2.3 | X-ray generator: minimum 140kV, orientation: vertically upwards / diagonal. |  |  |  |
| 2.2.2.4 | Wire resolution: minimum 40 AWG. |  |  |  |
| 2.2.2.5 | Steel penetration: minimum 35 mm. |  |  |  |
| 2.2.2.6 | Film safety: up to ISO1600. |  |  |  |
| 2.2.2.7 | Material separation: Low Z, Medium Z, High Z. |  |  |  |
| 2.2.2.8 | Luggage counter. |  |  |  |
| 2.2.2.9 | Date / time display. |  |  |  |
| 2.2.2.10 | Image archiving. |  |  |  |
| 2.2.2.11 | Zoom: minimum X8 |  |  |  |
| 2.2.2.12 | Free programmable keys; one key need to be programmed to show only organic materials. |  |  |  |
| 2.2.2.13 | View of previous bags: minimum 10 bags. |  |  |  |
| 2.2.2.14 | Sticker / sign indication the best orientation for small items shall be on the both ends of the machine with text “Place small items this side”. |  |  |  |
| 2.2.2.15 | Conveyer load: minimum 160kg. |  |  |  |
| 2.2.2.16 | Screening direction: 2 ways. |  |  |  |
| 2.2.2.17 | Input and output table for the x-ray machine length for the two sides shall be 1m (+/-10%) each. |  |  |  |
| 2.2.2.18 | Monitor shall be included with all necessary cables. Minimum 19 inches. |  |  |  |
| 2.2.2.19 | Operator keyboard shall be included with all the necessary cables. |  |  |  |
| 2.2.2.20 | STP test shall be passed to comply with EC and ECAC regulations without using any special image manipulation functions. |  |  |  |
| 2.2.2.21 | Possibility to add, make and manage own TIP images. |  |  |  |
| 2.3 | Maintenance |  |  |  |
| 2.3.1 | Recommended and planned daily, weekly, monthly and yearly maintenance schedule shall be presented with the offer indicating which maintenance need to be done by operator, client engineers and / or distributor / factory engineers. |  |  |  |
| 2.3.2 | List of scheduled preventive maintenance shall be indicated. |  |  |  |
| 2.4 | Spare parts & disposal auxiliaries |  |  |  |
| 2.4.1 | Recommended list of spare parts for one year of operational use shall be included. |  |  |  |
| 2..4.2 | Delivery time schedule for recommended spare parts shall be provided. |  |  |  |
| 2.5. | Manuals to be provided with the equipment |  |  |  |
| 2.5.1 | Operator manual shall be provided in English language. |  |  |  |
| 2.5.2 | Maintenance manual in shall be provided in English language, including technical description with diagrams and instructions for maintenance and repair. |  |  |  |
| 2.6. | Training |  |  |  |
| 2.6.1 | Price shall include technical maintenance training for technical personnel to qualify them to conduct service and maintenance at a level that enables to conduct weekly and yearly maintenance and problem shooting including changing x-ray tubes and belt tracking. |  |  |  |
| 2.7 | Warranty |  |  |  |
| 2.7.1 | Warranty period shall be 1 year from the date of equipment commissioning. The commercial warranty of at least two years including all spare parts and labour will be provided. |  |  |  |
| 3 | STANDARD TEST PIECE (STP) FOR CALIBRATING CONVENTIONAL X-RAY DEVICES. |  |  |  |
| 3.1. | Equipment description The STP is used to ensure that the performance requirements for X-ray machines laid down by domestic and international regulatory authorities – are met and maintained. |  |  |  |
| 3.2. | Technical specifications |  |  |  |
| 3.2.1 | STP is manufactured to an exact specification to comply with EC regulation 781/2005 and ECAC (DOC30 and annexes) requirements for regular testing of x-ray equipment. |  |  |  |
| 3.2.2 | The STP contains predefined, in EC and ECAC regulations (mentioned in 3.2.1), sets of metal pieces, wires and organic and inorganic material – that while screening the STP in the X-Ray machine, the operator is capable to diagnose the system’s sensitivity. At least following tests shall be able to be conducted with the STP: |  |  |  |
| 3.2.2.1 | Single wire resolutions; 30 AWG and 38 SWG un-insulated tinned copper wires, not covered by the aluminium step wedge. |  |  |  |
| 3.2.2.2 | Useful penetration; 24 AWG un-insulated tinned copper wires, behind 11.1 mm of the aluminium step wedge. |  |  |  |
| 3.2.2.3 | Spatial resolution; Vertical and horizontal 1.0 mm and 2.0 mm copper sheet gratings. |  |  |  |
| 3.2.2.4 | Single penetration (thin material imaging); 0.05 mm, 0.10 mm and 0.15 mm steel shim. |  |  |  |
| 3.2.2.5 | Single penetration (thick material differentiation); Lead strip behind 20 mm and 30 mm of steel. |  |  |  |
| 3.2.2.6 | Material discrimination; organic / inorganic. |  |  |  |
| 4 | CONVENTIONAL DUAL VIEW X-RAY SYSTEM FOR HOLD BAGGAGE SCREENING WITH TIP |  |  |  |
| 4.1 | Equipment description Detection systems that use x-ray generator to interrogate the material in question and detect the response of all present materials. |  |  |  |
| 4.2 | Technical specifications |  |  |  |
| 4.2.1 | Equipment shall comply with all requirements laid down by ECAC (DOC30 and annexes) and European Union (EC781/2005) for x-ray machines used to screen hold baggage. |  |  |  |
| 4.2.2 | In addition to ECAC and EU requirements mentioned at least following technical requirements shall be met: |  |  |  |
| 4.2.2.1 | Maximum object's size; 1000 mm * 1000 mm. |  |  |  |
| 4.2.2.2 | TIP system shall be installed (with optional capability to give individual access code for each user or to use single code for all operators according to the need of airport authority). |  |  |  |
| 4.2.2.3 | X-ray generator: minimum 140kv. |  |  |  |
| 4.2.2.4 | Wire resolution: minimum 36 AWG. |  |  |  |
| 4.2.2.5 | Steel penetration: minimum 27 mm. |  |  |  |
| 4.2.2.6 | Film safety: up to ISO1600. |  |  |  |
| 4.2.2.7 | Material separation: Low Z, Medium Z, High Z. |  |  |  |
| 4.2.2.8 | Luggage counter. |  |  |  |
| 4.2.2.9 | Date / time display. |  |  |  |
| 4.2.2.10 | Image archiving. |  |  |  |
| 4.2.2.11 | Zoom: minimum X8. |  |  |  |
| 4.2.2.12 | Free programmable keys; one key shall be possible to be programmed to show only organic materials. |  |  |  |
| 4.2.2.13 | View of previous bags: minimum 10 bags. |  |  |  |
| 4.2.2.14 | Sticker / sign indication the best orientation for small items shall be on the both ends of the machine with text “Place small items this side”. |  |  |  |
| 4.2.2.15 | Minimum conveyer load: 180kg. |  |  |  |
| 4.2.2.16 | Screening direction: 1 way. |  |  |  |
| 4.2.2.17 | Colour monitor shall be included with all necessary cables, minimum 17 inches. |  |  |  |
| 4.2.2.18 | Operator keyboard shall be included with all necessary cables. |  |  |  |
| 4.2.2.19 | STP test shall be passed to comply with EC and ECAC regulations (mentioned in 3.1.1.1) without using any special image manipulation functions. |  |  |  |
| 4.2.2.20 | System shall enable adding, making and managing own TIP images. |  |  |  |
| 4.3 | Maintenance |  |  |  |
| 4.3.1 | List of scheduled preventive maintenance shall be indicated. |  |  |  |
| 4.4 | Spare parts & disposal auxiliaries |  |  |  |
| 4.4.1 | Recommend list of spare parts for one year operational use shall be included. |  |  |  |
| 4.4.2 | Delivery time schedule for recommended spare part shall be provided. |  |  |  |
| 4.5 | Manuals that shall be provided with the equipment |  |  |  |
| 4.5.1 | Operator manual in English language. |  |  |  |
| 4.5.2 | Maintenance manual in English language, including technical description with diagrams and instructions for maintenance and repair. |  |  |  |
| 4.6 | Training |  |  |  |
| 4.6.1 | Technical maintenance training for technical personnel to qualify them to conduct service and maintenance shall be included at a level that enables to conduct weekly and yearly maintenance and problem shooting including changing x-ray tubes and belt tracking.. |  |  |  |
| 4.7 | Warranty |  |  |  |
| 4.7.1 | Warranty period shall be 1 year from the date of equipment commissioning. The commercial warranty of at least two years including all spare parts and labour will be provided. |  |  |  |

LOT No.3

| Item Number | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 5 | HHMD (HAND HELD METAL DETECTOR) |  |  |  |
| 5.1. | Equipment description A device that can be used manually to scan people from metal prior letting them to enter secured zone. |  |  |  |
| 5.2 | Technical specifications |  |  |  |
| 5.2.1 | Visual indicator to show that that the equipment is switched on. |  |  |  |
| 5.2.2 | Audible alarm (buzzer) and visual alarm (led light or similar). |  |  |  |
| 5.2.3 | Vibrating alarm. |  |  |  |
| 5.2.4 | Battery operated. |  |  |  |
| 5.2.5 | Shall detect both magnetic and non-magnetic metals. |  |  |  |
| 5.2.6 | The manufacturer shall show evidence that the delivered equipment is shock resistant, and stand daily use and minor drops. |  |  |  |
| 5.2.7 | Sensitivity of detecting area shall not have variation and must be uniform, no matter in which orientation and which part of detection area of HHMD is used. |  |  |  |
| 5.3 | Proof of compliancy on safety and security standards and regulations |  |  |  |
| 5.3.1 | Certification from competent authorized body indicating that equipment is safe for pregnant women, and wearers of pace makers or other life supporting equipment shall be provided with the offer. |  |  |  |
| 6 | WALK-THROUGH METAL DETECTOR GATE |  |  |  |
| 6.1 | Equipment description A device that generates an electromagnetic field for the detection of metallic items inside a monitored passage. |  |  |  |
| 6.2 | Technical specifications |  |  |  |
| 6.2.1 | Equipment shall comply with all requirements laid down by ECAC (DOC 30 and annexes) and European Union (EC1862/2006) for “walk through metal detectors” that are used to screen people and staff for unauthorized metal objects prior letting them to enter secured area. |  |  |  |
| 6.2.2 | In addition to ECAC and EU requirements mentioned at least following technical requirements shall be met: |  |  |  |
| 6.2.2.1 | Inner dimensions of the gate; minimum/maximum size 2020–2060 mm x 700–720 mm. |  |  |  |
| 6.2.2.2 | All access to settings shall be protected with password and physical lock. |  |  |  |
| 6.2.2.3 | Alarm: audible and visible alarm. |  |  |  |
| 6.2.2.4 | Random alarm: shall be programmable and %-rate must be changeable by airport technical staff. |  |  |  |
| 6.2.2.5 | Passenger counter; shall be programmable for 1 or 2 way passage. |  |  |  |
| 6.2.2.6 | Detector shall be able to be fixed on the ground without drilling holes on the floor. |  |  |  |
| 6.2.2.7 | Metal detector shall be operational in distance of 500-700 mm from x-ray machine without major interference. |  |  |  |
| 6.2.2.8 | Security level shall be displayed on start-up to allow users to verify correct level. |  |  |  |
| 6.2.2.9 | Detector shall be Multi Zone. |  |  |  |
| 6.2.2.10 | Shall be able to be calibrated to work with various detectors on distance of 200 mm. |  |  |  |
| 6.2.2.11 | Possibility to connect metal detector to network via LAN for distance data collection and programming. |  |  |  |
| 6.2.2.12 | In case the detector is connected to network and connection to LAN will be cut, all data recorded during the cut off, shall be stored inside the metal detector and shall be able to be recovered immediately, after connection to net has been recovered. Detector shall send data automatically without waiting for response from other equipment to give as accurate and up-to-date information as possible. |  |  |  |
| 6.2.2.13 | Fast security level changing if metal detector is connected to network supporting metal detectors and data collection shall be possible. |  |  |  |
| 6.3 | Proof of compliancy on safety and security standards and regulations |  |  |  |
| 6.3.1 | Certification from competent authorized body - indicating that equipment is safe for pregnant women, and wearers of pace makers or other life supporting equipment. |  |  |  |
| 6.4 | Maintenance |  |  |  |
| 6.4.1 | Recommended and planned daily, weekly, monthly and yearly maintenance schedule shall be presented with the offer indicating which maintenance need to be done by operator, client engineers and/or distributor/factory engineers. |  |  |  |
| 6.4.2 | List of scheduled preventive maintenance shall be indicated. |  |  |  |
| 6.5 | Spare parts & disposal auxiliaries |  |  |  |
| 6.5.1 | Recommended list of spare parts for one year operational use shall be included. |  |  |  |
| 6.5.2 | Delivery time schedule for recommended spare part shall be provided. |  |  |  |
| 6.6 | Manuals that shall be provided together with the equipment |  |  |  |
| 6.6.1 | Operator manual in English language. |  |  |  |
| 6.6.2 | Maintenance manual in English language, including technical description with diagrams and instructions for maintenance and repair. |  |  |  |
| 6.7 | Training |  |  |  |
| 6.7.1 | Technical maintenance training for technical personnel to qualify them to conduct service and maintenance shall be included at a level that enables to conduct, weekly, monthly and yearly maintenance and troubleshooting including programming and sensitivity changing, changing of defect parts such as coil panels and minimizing external disturbance.. |  |  |  |
| 6.8 | Warranty |  |  |  |
| 6.8.1 | Warranty period shall be 1 year from the date of equipment commissioning. The commercial warranty of at least two years including all spare parts and labour will be provided. |  |  |  |
| 7 | TEST PIECES FOR WALK THROUGH METAL DETECTORS |  |  |  |
| 7.1 | Equipment description Test pieces are used to ensure that the performance requirements for walk through metal detectors laid down by domestic and international regulator authorities - are met and maintained. |  |  |  |
| 7.2 | Technical specifications |  |  |  |
| 7.2.1 | The equipment shall comply with exact specification with EC regulation 1862/2006 (standard 2) and ECAC (DOC 30 and annexes) requirements for regular testing of “walk through metal detector". |  |  |  |
| 7.2.2 | The delivered kit shall include at least all knifes described on the standard 2 and the smallest gun part in the list for standard 2. |  |  |  |
| 7.2.3 | All items shall be marked with arrows to indicate at least 6 different orientations for testing. |  |  |  |

LOT No.4

| Item Number | Specifications Required | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 8 | ON-SITE TRACE CHEMICAL DETECTION Ion Mobility Spectroscopy (IMS) based technology. |  |  |  |
| 8.1 | Equipment description Stationary device that detect and identifies explosives and/or narcotics during a single analysis. Explosive and narcotics are detected by chemical identification of microscopic residues of the explosive / narcotic compound. These residues can be in either or both of two forms – vapour and particulate. |  |  |  |
| 8.2 | Technical specifications |  |  |  |
| 8.2.1 | All equipment shall comply with ECAC (DOC 30 and annexes) recommendations concerning trace detectors used securing civil aviation. In addition to ECAC requirements mentioned at least following technical requirements shall be met: |  |  |  |
| 8.2.2 | Technology shall be based on IMS (Ion Mobility Spectrometry). |  |  |  |
| 8.2.3 | Possibility for dual use (explosives / narcotics). |  |  |  |
| 8.2.4 | Warm up time: 30 minutes or less. |  |  |  |
| 8.2.5 | Analysis time: less than 10 seconds. |  |  |  |
| 8.2.6 | Sampling method: swab with / without wand or advanced. |  |  |  |
| 8.2.7 | Printer connection or built-in printer. |  |  |  |
| 8.2.8 | User friendly analysis feed back with clear indication if the item is cleared or not cleared. (Results should be stated clearly with text indication such as: PASS or CLEAR / HITT, ALARM or EXPLOSIVES or similar easy to understand text or with colours GREEN=PASS RED=ALARM). |  |  |  |
| 8.2.9 | Detects most common explosives such as; RDX, PETN, NG, TNT… and others indicated by ECAC (DOC 30 and annexes), ICAO (annex17). |  |  |  |
| 8.2.10 | Detect most common drugs such as but not limited to: Cocaine, heroin, amphetamine, THC, LSD, methamphetamine. |  |  |  |
| 8.2.11 | Calibrating and maintenance software shall be password protected. |  |  |  |
| 8.2.12 | Test traces (pens or samples) for explosives and drugs shall be provided with the equipment. |  |  |  |
| 8.2.13 | Type of the radioactive source shall be clearly indicated and marked. |  |  |  |
| 8.2.14 | User friendly daily maintenance and calibration without tools. |  |  |  |
| 8.2.15 | Easy to use control panel. |  |  |  |
| 8.3 | Maintenance |  |  |  |
| 8.3.1 | Recommended and planned daily, weekly, monthly and yearly maintenance schedule shall be presented with the offer indicating which maintenance is needed to be done by operator, client engineers and/or distributor/factory engineers. |  |  |  |
| 8.3.2 | List of scheduled preventive maintenance shall be indicated. |  |  |  |
| 8.4 | Spare parts & disposal auxiliaries |  |  |  |
| 8.4.1 | Recommend list of spare parts for one calendar year operational use shall be included. |  |  |  |
| 8.4.2 | Delivery time schedule for recommended spare part shall be provided. |  |  |  |
| 8.5.2 | Recommended list of consumables for one year operational use shall be included in the offer. |  |  |  |
| 8.5.3 | Delivery time schedule for recommended consumables shall be provided. |  |  |  |
| 8.6 | Manuals that shall be provided together with the equipment |  |  |  |
| 8.6.1 | Operator manual in English language. |  |  |  |
| 8.6.2 | Maintenance manual in English language, including technical description with diagrams and instructions for maintenance and repair. |  |  |  |
| 8.7 | Training |  |  |  |
| 8.7.1 | Technical maintenance training for technical personnel to qualify them to conduct service and maintenance shall be included at a level that enables to conduct, daily, weekly, monthly and yearly maintenance and troubleshooting, including cleaning cycles and spare parts changing. |  |  |  |
| 8.8 | Warranty |  |  |  |
| 8.8.1 | Warranty period shall be 1 year from the date of equipment commissioning. The commercial warranty of at least two years including all spare parts and labour will be provided. |  |  |  |
| 9 | HAND-HELD (PORTABLE) TRACE DETECTOR Ion Mobility Spectroscopy (IMS) based technology. |  |  |  |
| 9.1 | Equipment description Portable device that detect and identify explosives, narcotics and/or CW agents during a single analysis. Explosive and narcotics are detected by chemical identification of microscopic residues of the explosive / narcotic compound. These residues can be in either or both of two forms – vapour and particulate. |  |  |  |
| 9.2 | Technical specifications |  |  |  |
| 9.2.1 | All equipment shall comply with all ECAC (DOC 30 and annexes) recommendations concerning trace detectors used securing civil aviation. In addition to ECAC requirements mentioned at least following technical requirements shall be met: |  |  |  |
| 9.2.2 | Technology shall be based on IMS (Ion Mobility Spectrometry). |  |  |  |
| 9.2.3 | Possibility for multi use (explosives / narcotics / CW agents). |  |  |  |
| 9.2.4 | Warm up time; Max.10 minutes. |  |  |  |
| 9.2.5 | Analysis time; Max. 15 seconds. |  |  |  |
| 9.2.6 | Sample collection methods - option for at least two different ways: particle and vapour. |  |  |  |
| 9.2.7 | User friendly analysis feed back with clear indication whether the item is cleared or not cleared. |  |  |  |
| 9.2.8 | Detect most common explosives such as but not limited to; RDX, PETN, NG, TNT and indicated by ECAC (DOC 30 and annexes), ICAO (annex17). |  |  |  |
| 9.2.9 | Detect most common drugs such as but not limited to; Cocaine, heroin, amphetamine, THC, LSD, methamphetamine. |  |  |  |
| 9.2.10 | Detect most common chemical warfare agents including but not limited to nerve and blister agents such as sarin, tabun, agent VX, nitrogen mustard 3… |  |  |  |
| 9.2.11 | Calibrating and maintenance software shall be password protected. |  |  |  |
| 9.2.12 | Test traces (pens or samples) for explosives and drugs shall be provided with the equipment. |  |  |  |
| 9.2.13 | Type of the radioactive source shall be clearly indicated and marked. |  |  |  |
| 9.2.14 | Rechargeable battery which will operate the system minimum for 4 hours on regular use, including all necessary equipment for charging the battery. |  |  |  |
| 9.2.15 | Weight: under 4 kilos including battery. |  |  |  |
| 9.2.16 | User friendly daily maintenance and calibrating without tools. |  |  |  |
| 9.2.17 | Easy to use control panel. |  |  |  |
| 9.2.18 | Colour display |  |  |  |
| 9.3 | Maintenance |  |  |  |
| 9.3.1 | Recommended and planned daily, weekly, monthly and yearly maintenance schedule shall be presented with the offer indicating which maintenance need to be done by operator, client engineers and/or distributor/factory engineers. |  |  |  |
| 9.3.2 | List of scheduled preventive maintenance shall be indicated. |  |  |  |
| 9.4 | Spare parts & disposal auxiliaries |  |  |  |
| 9.4.1 | Recommend list of spare parts for one year operational use shall be included. |  |  |  |
| 9.4.2 | Delivery time schedule for recommended spare part shall be provided. |  |  |  |
| 9.4.3 | Recommended list of consumables for one calendar year operational use shall be included. |  |  |  |
| 9.4.4 | Delivery time schedule for recommended consumables shall be provided. |  |  |  |
| 9.5 | Manuals that shall be provided together with the equipment |  |  |  |
| 9.5.1 | Operator manual in English language. |  |  |  |
| 9.5.2 | Maintenance manual in English language, including technical description with diagrams and instructions for maintenance and repair. |  |  |  |
| 9.6 | Training |  |  |  |
| 9.6.1 | Technical maintenance training for technical personnel to qualify them to conduct service and maintenance shall be included at a level that enables to conduct, daily, weekly, monthly and yearly maintenance and troubleshooting, including cleaning cycles and spare parts changing. |  |  |  |
| 9.7 | Warranty |  |  |  |
| 9.7.1 | Warranty period shall be 1 year from the date of equipment commissioning. The commercial warranty of at least two years including all spare parts and labour will be provided. |  |  |  |